FHA Inspection Checklist (08/03)

Address: _______________________________   Date: __________________________

(Checked items will require repairs to meet HUD guidelines)

# VC-1 Site Hazards and Nuisances

( )	Other –gas/oil wells or slush pits

( )	High pressure gas or petroleum lines

( ) 	Overhead high voltage lines within fall distance

( )	Excessive smoke, fumes, offensive noises or odors

( )	New/proposed construction in flood hazard area

( )	Within 300 feet of stationary storage tanks >1000 gallons of flammable material

( )	Excessive noise or hazard from heavy traffic area

# VC-2 Soil Contamination

( ) 	Observable septic tank failure

( )	Surface evidence of Underground storage tank

( )	Proximity to dumps, landfills, industrial sites or locations that could contain hazardous materials

( )	Presence of pools of liquid, pits, ponds, stressed vegetation, stained soils or pavement, drums or odors

# VC-3	Grading and Drainage

( )	Grading does not slope away from home

( )	Standing water next to structure

# VC-4 Well, Individual Water Supply and Septic

( )	Property lacks connection to public water.  Distance to public lines?__________________

( )	Property lacks connection to public sewer.  Distance to public lines?__________________

( )	Septic – Type of system _________ Topography___________ Depth to Ground water_________

( )	Septic – Soil permeability _____________ Type of soil to a depth of several feet _____________

( )	Septic – Distance to the well _______________ Distance to property lines __________________

# VC-5 Wood Destroying Insects

( )	Structure and accessory buildings are ground level and/or wood is touching ground

( )	Signs of active termites

# VC-6 Private Road Access and Maintenance

( )	Property inaccessible by foot or vehicle

( )	Property accessible only by a private road or drive ______________(name of road/drive)

( )	Private road or drive not dedicated to public use and maintenance

( )	Private road or drive not protected by permanent recorded easements approved by HUD

( )	No all-weather surface (gravel is okay) describe road details ________________________

# VC-7 Structural Conditions

## Floor support Systems

( )	Significant cracks (take photos)

( )	Evidence of water leakage or damage?  Location:__________________________________

( )	Rodent infestation

## Framing/Walls/Ceiling

( )	Significant cracks

( )	Visible holes that could effect structure

( )	Significant water damage _____________________________________________________

## Attic

( )	Evidence of holes

( )	Support structure not intact or damaged

( )	Significant water damage visible from interior

( )	No ventilation by vent, fan, or window

( )	No attic access

# VC-8 Foundation

Foundation/Basement

( )	Inadequate access

( )	Evidence of significant water damage

( )	Significant cracks or erosion in exposed areas that could effect structural soundness

( )	Basement windows not at ground level or doesn’t have adequate egress space (24 X 36 minimum)

()	Basement windowsill cannot be more than 44 inches off the floor.

## Crawl Space

( )	Inadequate access

( )	Floor to bottom of floor joists less than 18 inches

( )	Support beams not intact

( )	Excessive dampness or ponding of water

( )	Trash or debris in crawl space (3-13)

( )	Inadequate ventilation

# VC-9 Roofing

( )	Does not cover entire house

( )	Evidence of deterioration of roofing materials

( )	Roof life less than two years

( )	More than three layers of roofing

( )	Holes

( )	Signs of leakage observable from the ground (missing tiles)

( )	flat roof (even if it’s a covered porch or carport except on condos)

( )	Roof obscured by snow

# VC-10 Mechanical Systems

## Furnace/Heating system

( )	Unit does not turn on

( )	Warm air not emitted

( )	Unusual or irregular noises are heard

( )	Smoke or irregular smell emitted

( )	Unit shuts down PRIOR to reaching desired temperature

( )	Significant holes or deterioration on the unit(s)

( )	System could not be operated properly due to weather (make note on V-10)

Air Conditioning (central)

( )	Unit does not turn on

( )	Cold air is not emitted

( )	Irregular noises are heard

( )	Smoke or irregular smell is emitted

( )	Unit shuts down PRIOR to reaching desired temperature

( )	Significant holes or deterioration on the unit(s)

( )	System could not be operated properly due to weather (make note on V-10)

## Electrical System

( )	Electrical switches do not turn on or off – test one per room ____________________________

( )	Outlets do not function – test one per room _________________________________________

( )	Presence of sparks or smoke from outlet(s)

( )	Exposed wiring visible in living areas

( )	Frayed wiring

( )	Missing switch and/or outlet plate covers

## Plumbing System

### Toilets

( )	Toilet(s) do not function

( )	Presence of leaks

( )	Missing separate utility service shut-offs

( )	Each unit doesn’t have individual meters

### Leaks

( )	Structural damage under fixtures

( )	Puddles present

### Sewer System

( )	Observable surface evidence of malfunction

### Sinks

( )	Basin or pipes leak

( )	Water does not run

### Water

( )	Significant drop or limitation in pressure

( )	No hot water

( )	If hot water heater is in living area, is it properly enclosed and vented

( )	If hot water heater is in non-living area, is it properly enclosed and vented

( )	No pressure relief valve on water heater

# VC-11 Other Health and Safety Deficiencies

( )	Broken window(s)

( )	Broken or missing exterior stairs

( )	Broken,  or missing exterior doors

( )	Inadequate/blocked entrances or exits

( )	More than two steps without handrails

( )	Mechanical garage door does not reverse when meets reasonable resistance

( )	Other health or safety hazards

( )	Non-operable windows (test at least one per room.  If Dbl hung, only one half needs to work)

( )	Smoke alarm not adjacent to sleeping areas (203k program only)

( )	Power lines (any!) over the primary living unit

( )	Improvements in the “fall line” of high-voltage transmission lines

( )	Other adverse or offensive odors or noises that threaten the health and safety of the occupants?

( )	Bedroom egress blocked or non-existent

( )	Missing gutters/downspouts

# VC-12 Lead Based Paint Hazard (if built PRIOR to 1978)

( )	Evidence on interior ___________________________

( )	Evidence on exterior ___________________________ (excluding condos)

( )	Paint chips left on the ground

# VC-13 Condominiums and Planned Unit Developments

( )	Project is not on FHA’s approval list

( )	Property does not meet owner-occupancy standards

( )	Property does not meet completion Standards

# Other

( )	if <13 months old, month and year of completion ________________

( )	Non-personalty items with less than 2 years Rem. Econ. Life _________________________

( )	Bars on windows do not comply with local fire codes.  Location:_______________________

( )	Design of subject “out-of-harmony” with the neighborhood

( )	Observable encroachments or non-typical easements (to or by the subject property)

( )	Observable soil contaminants

( )	Observable evidence of underground storage tanks

( )	Access to living unit is through another living unit

( )	Access to rear yard is through a different living unit

( )	Wood floor’s finish has worn off and exposes the bare wood (3-18)

( )	Peeling interior paint (any year – 3-18)

( )	Seriously cracked plaster or sheetrock (3/18)

( )	Carpet badly soiled to affect livability and/or marketability (Cleaning required 3-18)

( )	Excess land (larger than typical AND capable of a separate use) (4-5,6)

( )	Manufactured home without the tag

( )	Manufactured home without permanent concrete footings and tie-downs anchored to the footings

( )	Manufactured home built prior to June 15, 1976 (8-2)

( )	Manufactured home relocated from another site on-to current site

Note:  This is only a partially summary of items from the 4150.2 handbook

INTENDED USE : The intended use or function for all appraisals prepared for FHA is “to support the underwriting requirements for an FHA-insured mortgage”.

State of Colorado suggestion:  “Extraordinary Assumption and Limiting Condition:  As part of this assignment, the appraiser has completed the HUD/FHA Valuation Condition (VC) report form.  The purpose of this is to assist Direct Endorsement Underwriters in determining if the subject property meets minimum property standards for HUD/FHA mortgage insurance.  The VC checklist is based only on a visual inspection of the items listed.  The findings are based on conditions readily observable at the time of the appraisal inspection.  The appraiser is not a building contractor or qualified home inspector.  The appraiser is not qualified to survey or analyze physical items that are not easily visible.  If any parties to this transaction have concerns regarding structural, mechanical, infestation, contamination or other issues about the subject property, an expert in the appropriate field should be consulted.”